Human MI ReadyArray

49K Human Genomic Array

MI ReadyArray Human utilizes the whole genome probes developed by the Human Exonic Evidence Based Oligonucleotide (HEEBO) collaboration.  This optimized collection of 70mer probes was largely derived from constitutively expressed exons, and allows for the interrogation of nearly 25,000 human genes.  The exon-centric probe design can be used to differentiate alternatively expressed exons.  The HEEBO design also supports comparative genome hybridization (CGH) analysis. These arrays also contain an extensive assortment of controls that may facilitate accurate evaluation of expression results

| Gene-specific probes | 44,308 |
| --- | --- |
| Control probes | 4,650 |
| Total # probes | 48,958 |

For Research Use only

UPON RECEIPT OF YOUR ARRAYS

MI ReadyArrays are shipped in resealable desiccated storage packs.  These materials were specially chosen to reduce out-gassing of the packaging components.

Store arrays at room temperature.

To ensure maximum shelf-life and avoid high background levels, MI strongly suggests that newly received arrays and any unused arrays be stored in their original packaging including the outer metallic envelop.

PRODUCTION

MI precisely prints these 48,958 probes onto a single 1” x 3” standard-sized slide within HEPA filtered, environmentally controlled clean-rooms.  Our manufacturing processes utilize MI proprietary technologies and intellectual property licensed through Oxford Gene Technology.

All arrays are surface inactivated and processed to remove unbound DNA offer high DNA-binding capacity, deposition

uniformity and stability.  The MI ReadyArray surface has exceptionally low affinity for non-specific DNA attachment and resists various airborne and buffer-borne contaminants associated with high background levels.

QC FILES

The MI web site is your source for detailed quality control production data, an optimized sample labeling/hybridization protocol, and annotation and array design data available for every array purchased from MI.  Additional data regarding our exacting QC procedures can also be found there.

You may download the QC files for the specific array lot you receive from the MI website at www.microarrays.com under the QUALITY tab.  These data ensure your ability to readily identify the proper deposition of any probe of interest and to trace sources of variation due to lot-to-lot variation if your experiment spans multiple lots.

MI WARRANTEE

Every MI ReadyArray is guaranteed to have a minimum of 99.5% of the samples properly deposited and coupled to the array substrate.  The resultant QC data denotes the exact array location of any suspect depositions, i.e. beyond diameter specifications, irregular morphology, etc.  Any arrays not meeting these specifications will be replaced by MI.

### CONTACTING MI

For questions or support, please call 866.627.7297 (toll free) or email us at info@microarrays.com

Microarrays, Inc.

601 Genome Way, Suite 3100

Huntsville, AL 35801

1-256-327-0544

www.microarrays.com

HEEBO PROBE DATA

DESIGN APPROACH

Sequence Selection and Probe Design

Briefly, the pipeline for sequence selection and probe design included three steps:

- Collect and curate exon sequences, supplemented as needed with transcript sequences.

- Design and select the candidate 70mer probes for exon or transcript sequences.

- ArrayOligoSelector was used to generate a list of all candidate probes for each sequence.

- The probe list was filtered using a number of parameters (including probe uniqueness, self-binding score, complexity score, GC content) to narrow and rank the list of candidate probes.

- Optimal probes were chosen from the candidate list.  Probes had binding energies >-35 kcal/mol for other probe sequences, were less than 1,000 bases from the 3’ end of the transcript and would be present in all processed exons/transcripts.

When these three criteria could not be met, more than one probe was selected to represent the exon or transcript sequence.

OLIGONUCLEOTIDE SYNTHESIS AND QUALITY CONTROL

As part of the HEEBO collaboration between Stanford, Invitrogen, and Illumina, oligos were synthesized by Illumina on their proprietary Oligator® synthesis platform. Illumina utilizes multiple quality control methods to assess oligo quality including real time digital trityl monitoring, capillary electrophoresis, and OD260 analysis.  Further details regarding quality control of the HEEBO probes can be obtained through Invitrogen.

### CREDITS

MI, Illumina and Invitrogen wish to thank the following individuals for their effort in designing the probes on this array:

University of California, San Francisco: Mike Hagen, Catherine Foo, Jess Leber, David Erle, Jean Yang, Andrea Barczak, Joseph DeRisi, Jing Zhu

Stowers Institute: Chris Seidel

Stanford University: Pat Brown, Max Diehn, Kate Rubins, Stephen Popper, Joseph Marquis, Mike Fero, John Coller, Nicki Chin, Elena Seraia, Peng Zhang, Jon Pollack, Young Kim, Ash Alizadeh

Rockefeller University: Ben Snyder, Terry Gaasterland, Alexey Novoradovsky

University of Basel, Switzerland: Mihaela Zavolan

### ANNOTATION DETAILS

Information is provided in the MI_ HEEBO_GENELIST.XLS file found on the MI ReadyArray CD shipped with your arrays.

The file data include:

• Probe_Name: Contains the oligo name. The following codes can assist with interpreting the name:

- Rockefeller MouSDB3 constitutive exons/islands (names start with ‘scl’)

- LocusLink constitutive exons/islands  (names start with ‘scl0’)

- mRNA derived 70mers which may span intron/exon boundaries (names start with ‘scl00’)

- A collection of alternative spliced/skipped exons generated through extensive curation of 5 published datasets by Max Diehn, Ash Alizadeh, Jean Yang, and Catherine Foo (oligo names start with ‘scl000’)

- Syntenic orthologs of human loci exhibiting cis-antisense transcription based on Yelin et al Nature Biotech 2003. (names start with ‘scl0000’)

• LocusLink_ID: LocusLink ID number.

• Gid: GI number.

• Accession: Accession number.

• Symbol: Gene symbol.

• Probe Sequence: 70mer oligonucleotide probe sequence.

• Tm:  Oligo melting temperature.

• GC: Percentage of GC content within the probe sequence.

• Product: Probe target definition.

• Description: The specific annotation for each 70mer is detailed and is carat (^) delimited as follows for MCC plates:

- MCC Details: [LLID (060504) ^ Design type ^ cluster ^ Long Oligoname ^ outputset ^ worstxhyb ^ oligo_3_marg]

- Where LLID refers to LocusLink ID, which can be linked with gene annotation data through a variety of tools including BatchSource.

PROBE DETAILS

| NUMBER of PROBES | PROBE TYPE | PROBE FUNCTION |
| --- | --- | --- |
| 19,207 | Constitutive Exonic | Recognize all known transcripts of a human gene |
| 8,441 | Alternatively Spliced / Skipped Exonic | Recognize exons present in some, but not all transcripts of a human gene |
| 2,149 | Human ESTs | Recognize human EST sequences |
| 11,511 | Human mRNA | Recognize human mRNA sequences |
| 843 | BCR/ TCR Genic/ Regional | Recognize transcripts from human genes that undergo somatic rearrangement |
| 102 | Transgenic / Vector | Recognize elements commonly used for transgenic constructs (e.g. GFP, YFP). |
| 1,677 | Viral Probes | Recognize human viral pathogen sequences |
| 378 | Human microRNA Tagged Templates | Recognize targeted human microRNAs |
| 864 | Positive Controls | Recognize a small subset of human exons or transcripts |
| 194 | Negative Controls | Empty wells and 97 random sequences are positioned throughout the set to assist in determining background. |
| 2,733 | Spiking Controls | Recognize non-human sequences that can be spiked into RNA samples. |
| 643 | Other Controls | Recognize human repeats from SINEs and LINEs |